Since the model now doesn’t support flashing BIOS under DOS so please try to boot to USB in UEFI shell.

- Extract “EFI” folder and put it to USB

- Put the BIOS file + AFUE tool in the same folder and put it to USB

- Boot up the system and press F11 to go to boot menu

- Select the USB in UEFI mode

- Go to the USB by typing for example fs0:  (fs0 titik dua)

- Select the folder with AFUE tool + the BIOS file by typing cd FOLDER NAME and press enter

- Type AFUE592.efi EB183IMH.820 or Flash.nsh and press enter

- BIOS will start to flash